TechNova Solutions
Basic IPv4 Network Design & Implementation

GNS3 Lab Documentation — Level 1 (No VLANs / No ACLs)

# 1. Project Overview

This project documents the design and implementation of a basic IPv4 enterprise network for TechNova Solutions. The lab focuses on subnetting, router interface configuration, DHCP assignment, and basic connectivity testing. VLANs, inter-VLAN routing, ACLs, and advanced access-control mechanisms are intentionally excluded from this first version and are planned as future enhancements.

# 2. Company Requirements

Four departments: HR, Finance, IT, and Sales.

HR requires approximately 20 hosts.

Finance requires approximately 30 hosts.

IT requires approximately 50 hosts.

Sales requires approximately 60 hosts.

The company also has servers, printers, and network-management devices planned for a separate infrastructure network.

Department PCs should receive their IPv4 configuration through DHCP.

All departments are allowed to communicate with each other in this Level-1 implementation.

VLANs and access-control policies are future enhancements.

# 3. Base Network

Assigned IPv4 network: 192.168.10.0/24

The /24 network was divided using VLSM-style subnetting according to the host requirements. The department mapping below follows the original design used in the GNS3 implementation.

# 4. Subnetting / IP Addressing Plan

| Department | Network | CIDR | Mask | Usable Range | Broadcast | Gateway |
| --- | --- | --- | --- | --- | --- | --- |
| HR | 192.168.10.0 | /27 | 255.255.255.224 | 192.168.10.1–30 | 192.168.10.31 | 192.168.10.1 |
| Finance | 192.168.10.32 | /27 | 255.255.255.224 | 192.168.10.33–62 | 192.168.10.63 | 192.168.10.33 |
| IT | 192.168.10.64 | /26 | 255.255.255.192 | 192.168.10.65–126 | 192.168.10.127 | 192.168.10.65 |
| Sales | 192.168.10.128 | /26 | 255.255.255.192 | 192.168.10.129–190 | 192.168.10.191 | 192.168.10.129 |
| Reserved / Infrastructure | 192.168.10.192 | /26 | 255.255.255.192 | 192.168.10.193–254 | 192.168.10.255 | Planned |

Note: Network and broadcast addresses are not assignable to hosts. The first usable address in each department subnet is used as the router gateway.

# 5. GNS3 Topology

The initial implementation uses one Cisco IOSv router and four Layer-2 switches, with one switch representing each department. This avoids VLANs at the current learning stage and allows each subnet to connect to a dedicated router interface.

R1
              ___________|____________
             /        /      \         \
            /        /        \         \
         Gi0/0    Gi0/1      Gi0/2     Gi0/3
           |        |          |         |
        Switch5  Switch4    Switch3   Switch2
           |        |          |         |
         Sales      IT       Finance      HR
          PC3       PC4        PC2       PC1

The original single-switch concept is useful for understanding enterprise topology, but with five separate IP networks and no VLANs, a single physical switch-to-router link cannot provide separate Layer-3 networks without additional mechanisms. For this Level-1 lab, dedicated router interfaces are used for the four department networks.

# 6. Devices Used in GNS3

| Role | GNS3 Device | Quantity | Purpose |
| --- | --- | --- | --- |
| Router | Cisco IOSv | 1 | Layer-3 routing, gateway, DHCP server |
| Switches | Cisco IOSvL2 | 4 | Layer-2 connectivity for departments |
| PCs | VPCS | 4+ | End-host testing and DHCP clients |

# 7. Router Interface Plan

| Interface | Department | IP Address | Subnet Mask |
| --- | --- | --- | --- |
| Gi0/0 | HR | 192.168.10.1 | 255.255.255.224 |
| Gi0/1 | Finance | 192.168.10.33 | 255.255.255.224 |
| Gi0/2 | IT | 192.168.10.65 | 255.255.255.192 |
| Gi0/3 | Sales | 192.168.10.129 | 255.255.255.192 |

# 8. Router Configuration

Enter the following commands on R1:

enable
configure terminal

interface GigabitEthernet0/0
 ip address 192.168.10.1 255.255.255.224
 no shutdown
 exit

interface GigabitEthernet0/1
 ip address 192.168.10.33 255.255.255.224
 no shutdown
 exit

interface GigabitEthernet0/2
 ip address 192.168.10.65 255.255.255.192
 no shutdown
 exit

interface GigabitEthernet0/3
 ip address 192.168.10.129 255.255.255.192
 no shutdown
 exit

end
write memory

# 9. Verify Router Interfaces

show ip interface brief

All four connected interfaces should show an IP address with Status = up and Protocol = up. If an interface is administratively down, enter the interface configuration mode and use no shutdown.

# 10. DHCP Design

The Cisco IOSv router is used as the DHCP server. Department PCs receive IP address, subnet mask, default gateway, and DNS server automatically.

DHCP exclusions reserve the gateway and a small range of addresses for future static devices.

ip dhcp excluded-address 192.168.10.1 192.168.10.10
ip dhcp excluded-address 192.168.10.33 192.168.10.42
ip dhcp excluded-address 192.168.10.65 192.168.10.74
ip dhcp excluded-address 192.168.10.129 192.168.10.138

# 11. DHCP Pool Configuration

ip dhcp pool HR
 network 192.168.10.0 255.255.255.224
 default-router 192.168.10.1
 dns-server 8.8.8.8
 exit

ip dhcp pool FINANCE
 network 192.168.10.32 255.255.255.224
 default-router 192.168.10.33
 dns-server 8.8.8.8
 exit

ip dhcp pool IT
 network 192.168.10.64 255.255.255.192
 default-router 192.168.10.65
 dns-server 8.8.8.8
 exit

ip dhcp pool SALES
 network 192.168.10.128 255.255.255.192
 default-router 192.168.10.129
 dns-server 8.8.8.8
 exit

end
write memory

# 12. Configure VPCS Clients

On each VPCS client, request an address using DHCP:

ip dhcp
show ip

Expected subnet assignment:

HR PC → 192.168.10.0/27, gateway 192.168.10.1

Finance PC → 192.168.10.32/27, gateway 192.168.10.33

IT PC → 192.168.10.64/26, gateway 192.168.10.65

Sales PC → 192.168.10.128/26, gateway 192.168.10.129

# 13. DHCP Verification

On R1, verify that clients have received leases:

show ip dhcp binding
show ip dhcp pool
show running-config | section dhcp

A successful DHCP binding should show the client IP address, hardware/client identifier, lease information, and Active state.

Example observed DHCP leases during testing included 192.168.10.2, 192.168.10.3, 192.168.10.34, 192.168.10.66, and 192.168.10.139. These addresses correspond to the HR, Finance, IT, and Sales subnets respectively and confirmed that DHCP was functioning.

# 14. Connectivity Testing

Test the network in stages.

## 14.1 PC to Local Gateway

HR PC:
ping 192.168.10.1

Finance PC:
ping 192.168.10.33

IT PC:
ping 192.168.10.65

Sales PC:
ping 192.168.10.129

## 14.2 Inter-Department Routing

Test connectivity between hosts in different subnets. For example:

Sales PC → IT gateway
ping 192.168.10.65

Sales PC → Finance gateway
ping 192.168.10.33

Sales PC → HR gateway
ping 192.168.10.1

Because no ACLs or isolation policies have been configured in this version, the departments are expected to be able to communicate through the router.

# 15. Troubleshooting Commands

show ip interface brief
show ip route
show ip dhcp binding
show ip dhcp pool
show running-config
show running-config | section dhcp
ping <destination-ip>
traceroute <destination-ip>

# 16. Common Problems and Checks

| Problem | Check / Fix |
| --- | --- |
| Interface down | Check cable/connection and use no shutdown on the router interface. |
| PC receives no DHCP address | Check the correct DHCP pool network/mask, interface status, and physical link. |
| Wrong IP subnet | Compare the VPCS address with the subnet table and DHCP pool. |
| Gateway ping fails | Check the PC gateway, router interface IP, interface status, and cable. |
| Inter-subnet ping fails | Check show ip route and verify all router interfaces are up/up. |
| DHCP pool has wrong network | The network command must exactly match the intended subnet. |
| Excluded-address command rejected | Use ip dhcp excluded-address in global configuration mode, not inside dhcp-config mode. |

# 17. Infrastructure Network — Planned, Not Implemented Yet

The original /24 leaves the following address block unused after the four department networks:

192.168.10.192/26
Usable: 192.168.10.193 – 192.168.10.254
Broadcast: 192.168.10.255

This remaining subnet can later be used for servers, printers, switch management, and other infrastructure devices. For the current no-VLAN implementation, it is intentionally left for a later design stage rather than forcing additional complexity into the first lab.

# 18. Why VLANs Are a Future Enhancement

At the current stage, separate router interfaces are used to connect the four department networks. In a typical enterprise environment, VLANs can logically separate departments while using fewer physical router interfaces. Inter-VLAN routing can then provide communication between VLANs, and ACLs can control which departments are allowed to communicate.

HR VLAN → HR subnet

Finance VLAN → Finance subnet

IT VLAN → IT subnet

Sales VLAN → Sales subnet

Server VLAN → Infrastructure/server subnet

Management VLAN → switch and network-device management

# 19. Future Enhancements

Implement VLANs for HR, Finance, IT, Sales, Servers, and Management.

Configure inter-VLAN routing using router-on-a-stick or a Layer-3 switch.

Implement ACLs to restrict department-to-department communication.

Separate servers and printers into dedicated VLANs/subnets.

Configure switch management and management VLAN.

Add a firewall and Internet edge.

Add redundancy using additional switches/routers.

Introduce DNS/NTP services and centralized logging.

Add network monitoring and security controls.

# 20. Learning Outcomes

Converted host requirements into IPv4 subnet sizes.

Designed a basic VLSM-style addressing plan.

Configured router interfaces as default gateways.

Configured Cisco IOS DHCP pools.

Configured VPCS clients using DHCP.

Verified DHCP leases and router interfaces.

Tested local gateway and inter-subnet connectivity.

Identified why VLANs become useful as the network grows.

# 21. Final Project Summary

The completed Level-1 GNS3 lab demonstrates a functional IPv4 network for four departments. The 192.168.10.0/24 network was divided into four department subnets, router interfaces were configured as gateways, and DHCP was implemented on R1. VPCS clients successfully received addresses from the correct DHCP pools. The design intentionally avoids VLANs and ACLs so that the fundamentals of subnetting, routing, DHCP, and troubleshooting are established first.

# 22. Suggested GitHub Repository Structure

technova-basic-network/
├── README.md
├── docs/
│   └── Network_Design_and_Implementation.docx
├── topology/
│   └── gns3-topology.png
├── configs/
│   └── R1-config.txt
└── screenshots/
    ├── topology.png
    ├── dhcp-binding.png
    └── connectivity-tests.png

# 23. GitHub README Short Description

Basic enterprise network design and implementation in GNS3 using Cisco IOSv and IOSvL2. The project covers IPv4 subnetting, router interface configuration, DHCP, VPCS client configuration, and connectivity testing for HR, Finance, IT, and Sales departments. VLANs, inter-VLAN routing, and ACLs are planned as future enhancements.